## Motivation

Industrial heat is a dominant player in global emissions and industrial decarbonization needs to be part of the solution reduce emissions impacts.

There is substantial literature examining the cost of heat electrification at a sector wide and regional wide analysis. (Show some proof of what the impact in cost and emissions reduction). But there is a gap in facility level analysis and analysis on how many facilities would be able to cost effectively and competitively with gas boiler.

Industrial heat battery is a battery that stores energy as heat often using cheap abundant materials such as silica, ceramics or graphite. This storage technology can take renewable electricity during cheap hours of the day and discharge constant heat as steam for industrial purposes.

## Methods — outline

### 2.1 Overview and analytical framework

The goal of this research is to assess which EU industrial sites could cost-effectively replace fossil powered heat with solar + industrial heat battery (IHB) systems. To screen facility candidates, we started with climate trace open-source data of facilities in the European Union. The main methodology starts with bottom-up estimates of heat used in the manufacturing process. These bottom-up estimates are validated by country and sector level totals from Joint Research Centre's Integrated Database of the European Energy System (JRC-IDEES)/IndustryHeat-EU (Gjorgievski et al., 2025) to verify that useful heat is within acceptable range. The bottom-up, top-down verified, approach to heat estimates addresses the facility-level industrial decarbonization analysis gap that is missing in current literature. This method provides assurance that the bottom-up estimates are reasonable.

To complete the cost calculations, we gathered the land availability and solar capacity and put them as constraints in the flatblock optimization modeling. We used optimization methods to gather the optimal sizing of solar+IHB system the LCOH per site. With the LCOH calculated, facilities are analyzed to see which facilities are feasible and cost effective against gas generated heat and what characteristics would make these specific sites successful.

### 2.2 Facility dataset and scope

Climate Trace has 2,493 in EU-27 (excluding Malta, no facilities) plus the United Kingdom. After filtering out duplicate sites and correcting invalid data, there are 2,415 facilities used in this study. There are 11 subsectors for manufacturing analysis: food & beverage & tobacco, textiles & apparel, glass, cement, iron & steel, pulp & paper, lime, chemicals, other metals, petrochemical steam cracking, aluminum. For each facility, there are coordinates, annual production output, capacity factor, annual emissions in CO2. These facilities utilize heat temperatures from 100°C to 1500°C which could be decarbonized with heat pump or IHB.

### 2.3 Replaceable heat demand estimation

For each subsector, we gathered the useful heat demand that IHB can serve based on the specific energy consumption(Pezzutto et al., n.d.) and process efficiencies (Appendix 1). From useful heat, we calculated the replaceable heat by applying fossil share for heat by country and sector(Disaggregated Final Energy Consumption in Industry by NACE Rev. 2 Activity - Quantities, n.d.). The useful heat calculation is used for validation against JRC annual heat data, and the replaceable heat is the portion of heat used that is generated by fossil fuels which can be decarbonized by IHB.

| Calculation route | Formulas |
| --- | --- |
| SEC route | fuel_energy = production × SEC/1000 useful_heat = fuel_energy * η_process |
| Emissions route | fuel_energy = (CO₂e × combustion_share) / emissions_factor useful_heat = fuel_energy × η_boiler |
| Replaceable Calculations | replaceable_heat = useful_heat × fossil_share |

The production data for each sector varies and we developed a 3-tiered system to calculate the heat demand based on the best available data. The more reliable tier uses climate trace production data with SEC. The lower tier uses emissions to calculated useful heat.

#### 2.3.1 SEC Heat Estimates

For sectors that provide production annual totals per facility, we applied the SEC value to get the useful heat amount and applied the fossil share with the SEC value to get the replaceable heat. Heat-intensity table with selected values and sources are taken from Fraunhofer and noted in the references. The process efficiencies are gathered from different trusted sources and applied to the fuel energy to get the useful heat.

#### 2.3.2 Emissions Heat Estimates

For sectors that do not provide production amounts per facility, we used emissions to get the heat demand estimates. There are only two sectors that uses emissions to get the heat demand: food and textiles. In food and textile sector, the combustion emissions are entirely from fuel combustion unlike sectors such as cement which releases emissions from limestone calcination, not fuel. From the combustion emissions, we used an assumed gas/oil blend (EU reported mix of 90% gas/10% oil) to get the TJ of fuel and applied a boiler efficiency to get the useful heat.

Climate Trace have flagged the high uncertainty of the modeled estimates of emissions. When comparing the estimated energy need of different countries with JRC data(Gjorgievski et al., 2025), there are countries that are 18x overstated (Portugal) but also countries that are 0.4x (Finland) understated. To minimize the over/understatement, we use the ratio of Climate Trace emissions to JRC data per country and rescaled the emissions based on the ratio. It is important to note that food sector, and food sector alone is fitted to the validation data and therefore cannot be validated. However, this method still provides a reasonable sector-wide analysis for the food and beverages sector. The textile sector does not need to rescale since it is within a reasonable range of the validation data but still relies on emissions data.

#### 2.3.3 Fossil Share to replaceable heat

Fossil shares are taken from Eurostat data, matching NACE activity with Climate Trace sector categories and getting the fossil percentage per sector. Electricity and derived heat excluded because useful heat is combustion derived. IHB cannot replace heat produced by byproducts such as biomass in pulp and paper because biomass will still be burned as part of the chemical feedstock need to manufacture pulp and paper. Taking only the fossil share gives a more realistic accounting of what types of heat generation can be replaced by IHB.

fossil_share = fossil_fuel / (fossil_fuel + biomass_fuel)

### 2.4 Validation of the bottom-up estimate

We compared industrial process heat from our calculated bottom-up figures with the reference figures from JRC-IDEES/IndustrialHeatEU. From this dataset, we used the useful energy demand dataset which is the amount of energy that is used in the process. Specifically, we pulled the steam, non-electric process heat <100°C, non-electric process heat 100–400°C, non-electric process heat 400–1000°C, and non-electric process heat >1000°C at the country level and at the sector level to compare with our useful heat estimates. We matched the IndustrialHeatEU subsectors categories to the Climate Trace sector categories, which is shown in the appendix.

Overall, the facility bottom-up reproduces the country picture well. The EU-26 covered-sector total is 556 TWh vs 794 TWh reference (70%), and the largest industrial countries all land in a plausible band: DE 71%, FR 82%, ES 73%, IT 58%, PL 68%, NL 83%. The calculated totals are slightly under the reference totals which is explainable because Climate Trace does not account for all the facilities. Overall, the calculated totals do not go over 50% of the reference useful energy.

The sector-level totals also match the reference well except for pulp & paper and ceramics & glass. For pulp & paper, Climate TRACE lists 79 pulp mills covering 21.8 Mt of the 73.9 Mt EU pulp production (CEPI, 2023), but none of the 716 separate paper & board mills (CEPI, 2023). Because the IndustryHeat-EU reference spans the whole pulp-and-paper sector, pulp and paper-drying steam, the bottom-up captures 27% of its heat, the shortfall being the uncovered paper sector. For ceramics & glass, Climate TRACE lists only glass facilities, covering ~55% of EU glass production 20 of ~37.5 Mt (GAE STATISTICAL REPORT 2025-2026, n.d.). Glass itself matches reasonably. The reference column also bundles ceramics, brick and refractories which is ~85% of its heat, mostly >1000 °C kiln firing, that we do not model. (check and verify this last statement)

Because each country's food total is fitted to the JRC reference, the food estimate cannot be independently validated against it. The emissions data only set the within-country allocation. Textiles keeps its emissions-derived level and so remains an independent (if emissions-dependent) estimate.

### 2.5 Heat generation and load profile

#### 2.5.1 System Configuration

Each facility is modelled as a single on-site solar PV array feeding two independent, temperature-split technology chains: industrial heat pump and industrial heat battery:

The >200 °C IHB chain: solar electricity → IHB

The <200 °C heat-pump chain: solar → heat pump → warm store

| Heat demand band | Technology | COP | Notes |
| --- | --- | --- | --- |
| <100°C | Industrial heat pump (low-temperature) | 2.7 |  |
| 100–200°C | Industrial heat pump (steam) | 1.8 | Upper limit of the heat-pump pathway (~200°C) |
| >200°C, up to ~1,500°C | Industrial heat battery | 1.0 | No efficiency multiplier |

Every facility therefore uses whichever chains its temperature profile requires. A food plant is served almost entirely by heat pumps, a cement plant almost entirely by the resistive battery.

#### 2.5.2 Flat Load Assumption

Heat demand is estimated only at annual resolution (production × specific energy consumption × fossil share); no facility-level hourly or seasonal load data are available from Climate TRACE or the underlying benchmarks. Replaceable heat is therefore modelled as a flat, constant hourly load equal to the annual replaceable heat divided by 8,760 hours. This implies no diurnal ramping and no seasonal variation, and it treats average load as peak load for sizing purposes. It is a deliberate simplification driven by the annual aggregation step.

#### 2.5.3 Heat Demand by Temperature and Mapping to Technology

Each facility's heat is split across five temperature bands (<100, 100–200, 200–500, 500–1,000, and >1,000 °C) using process recipes drawn from industry benchmarks (Pezzutto et al., n.d.). Every benchmark process carries both a heat intensity and a characteristic temperature profile, so a facility's total heat is distributed across bands according to the actual thermodynamics of the processes it runs (e.g. cement clinker calcination falls mostly above 1,000 °C, textile finishing mostly below 200 °C). Technology is then assigned band by band. The two sub-200 °C bands to the heat pumps and all higher bands to the resistive heat battery. A single facility is typically served by a mix of both. Across the mapped fleet, this places about a quarter of replaceable heat (the <200 °C bands) on efficient heat pumps and about three-quarters (>200 °C) on the resistive thermal battery, which is the dominant driver of both cost and the required solar and land.

This is the general breakdown of the temperature bands:

E_replaceable, band_b  =  replaceable_heat_mwh  ×  ( heat_band_b / Σ heat_band )

Then each band becomes its own flat load stream:

load_b (MW)        = E_replaceable,b / 8760

### 2.6 Solar Radiation per Facility

Hourly photovoltaic output for each facility is retrieved from the PVGIS seriescalc API using the PVGIS-ERA5 reanalysis database for the calendar year 2023. Each site is modelled as a 1 kWp system so output is normalized per installed kilowatt-peak; panel tilt is set equal to the site's latitude, azimuth to due south (0° in the Northern Hemisphere, 180° in the Southern), and system losses to zero, giving an idealized resource to which losses can be applied later in sizing. The returned hourly AC power is converted to kW per kWp, its UTC timestamps are converted to the local timezone at each coordinate (via timezonefinder), floored to the hour, de-duplicated, and reindexed to a complete 8,760-hour local series with any missing hours set to zero. Two summary metrics are then derived per facility: the annual yield (the sum of the hourly series, in kWh per kWp per year) and the capacity factor (annual yield / 8,760).

Across the mapped facilities the annual yield has a median of 1,333 and a mean of 1,412 kWh/kWp, with every site falling inside the ~900–2,000 kWh/kWp band expected for European latitudes under the stated tilt and loss assumptions. Yield correlates strongly and negatively with latitude (Pearson r ≈ −0.92), and the spatial pattern reproduces the known European solar map — bright across Iberia, Italy and Greece, dark across the Nordics, the UK and the Baltics. The hourly profiles show the correct physical shape: a midday-peaked diurnal cycle with zero output overnight and a summer-peaked seasonal cycle. Representative sites span the resource range, from Cyprus (~2,049 kWh/kWp) through central France (~1,318) to northern Sweden at 67 °N (~967). These profiles supply the solar generation used to size the colocated PV against each facility's heat load, with land-limited capacity set by an assumed utility-scale power density.

### 2.7 Land availability for colocated solar

GEE zonal statistics; 5 km point buffer (15 km reserved for sensitivity); ESA WorldCover at 30 m.

Suitability mask: slope > 5° (SRTM + Copernicus DEM above 60°N), WDPA + 100 m buffer.

Available land = bare/sparse + cropland; exclusions. Screening-metric caveats.

Land availability is a cap in the optimization. If the full load with 90% reliability and land cap doesn’t solve, then the floor is dropped, unserved is penalized and achieved reliability percentage is given.

Each buffers every point, masks out slope > 5° and WDPA protected areas, and sums ESA World Cover area by land-cover class. The 15 km run is ~9× the pixels, so it takes noticeably longer.

developable / excluded-cover / slope+protected split shown; cropland+grassland dominate developable

available land median 24.4, mean 27.2 km²; 26 zero-land sites flagged

For each facility, we computed the land physically available for co-located solar within a 5 km buffer of its coordinates. A 15 km buffer is also computed and held for sensitivity analysis. Land cover comes from Esa World Cover at 30 m resolution using Google Earth Engine zonal statistics. Two exclusions are applied before summing developable area: slop greater than 5°C and WDPA-protected areas plus a 100m buffer around them. The remaining bare, cropland, grassland, and shrubland is summed as available land (km^2). Forest, built up land, water, wetland, snow/ice, mangroves, moss/lichen are set as not developable for solar. Available land is capped in the optimization. If a full flat load at 90% reliability threshold is not met within the land cap, then the model relaxes the reliability floor and reports whatever reliability was achievable within the site’s footprint.

All 2,415 facility carry land data and average 48.2 KM^2 per facility, 61% of the possible area in 5 km radius buffer. The remaining 30.3 km^2 is either masked out by slope/protected filters or falls outside WorldCover’s coastal/border coverage. 26 facilities have zero developable land and they concentrate in food, beverage & tobacco, textiles & apparel and pulp & paper sites. These are typically dense or built-up sites that have no open ground within 5km.

### 2.8 Technology and cost data

Country-level Bloomberg inputs (solar LCOE, IHB/storage, CCGT). Specify vintage, currency, discount rate, project lifetime, IHB round-trip efficiency — all currently TODOs and required for a methods section.

### 2.9 LCOH and system sizing (the biggest gap — currently unwritten)

Solar array + IHB sizing algorithm (hourly matching of flat load to PVGIS output; storage sizing logic).

LCOH equation for the renewable path; LCOH equation for the CCGT baseline.

Land constraint binding vs. non-binding.

Optimization constraints: list it out.

Usable storage fraction = 1 – T_max/T_min

current solver                 heat-pump extension (per band b)

──────────────                 ────────────────────────────────

S_MW  (solar)                  S_MW              ← shared across all bands

P_charge, P_discharge          P_charge_b, P_discharge_b

E_TES                          E_TES_b

load (single)                  load_b = band allocation / 8760

eta_e2h ≈ 0.97 (resistive)     COP_b = 2.7 (<100°C), 1.8 (100–200°C), 1.0 (>200°C)

The hourly constraints, per band:

- Solar allocation (the coupling): each hour, the shared array's output is split across the band converters — Σ_b elec_b[t] ≤ S_MW × cf_solar[t].

- Charge (this is where COP enters): heat into band b's store = elec_b[t] × COP_b × η_charge. The COP multiplier is the entire heat-pump effect.

- Storage dynamics & delivery: soc_b[t] = soc_b[t−1] + charge_b[t] − discharge_b[t]/η_discharge; discharge_b[t] + unserved_b[t] = load_b.

- Land cap (shared): S_MW ≤ available_land × 50.

- Objective: minimize [solar capex·S_MW + Σ_b(converter capex_b + storage capex·E_TES_b) + unserved penalty] / total annual load, at the 90% availability target.

## 3. Analysis

### 3.1 Site screening and candidate classification

Decision criteria (solar+IHB LCOH < CCGT LCOH; minimum land / yield thresholds).

Aggregate outputs: replaceable heat, candidate count/share, LCOH-delta distribution, emissions reduction.

### 3.2 Sensitivity analysis

One-at-a-time (and combined) variation: HEAT ESTIMATE, fossil share, heat intensity, tech costs, IHB efficiency, solar year, land classification, capacity factor. Output format (tornado / site-count curves).

### 3.3 Limitations

Sector-average intensity & fossil share; flat-load assumption; point-buffer land screening; country-level (not site-level) costs; Climate TRACE coverage bias (floor estimate).

## Appendix

- Subsector Process Efficiencies (fuel input to useful heat)

| Subsector | eff | What it represents | Source |
| --- | --- | --- | --- |
| Cement | 0.60 | Kiln pyro processing first-law eff | Madlool et al. 2011 (Renew. Sustain. Energy Rev. 15:2042), 50–60% kiln-system; cross-checked vs IEA Cement Roadmap (BAT ~3.0 vs theoretical ~1.8 GJ/t) |
| Lime | 0.65 | EU mix of shaft (~0.80) + rotary (~0.55) kilns | EU BREF Cement/Lime/MgO (4–5 GJ/t vs theoretical ~3.2) |
| Glass | 0.45 | Melting-furnace eff (large flue-gas/structural losses) | Beerkens 2008 (glass-furnace energy benchmarking); Schmitz et al. 2011 (regenerative melting 40–50%) |
| Iron & steel | 0.70 | Reheating/sinter furnaces w/ recuperation (65–75%) | IEA Iron & Steel Roadmap; worldsteel energy statistics |
| Petrochem steam cracking | 0.90 | Fired heater w/ strong convection recovery | Ren, Patel & Blok 2006 (Energy 31:425); Ullmann's "Ethylene" — flagged least-certain |
| Chemicals — ammonia/methanol | 0.90 | Reformers / fired heaters w/ heat recovery | (same class as cracking) |
| Chemicals — soda ash | 0.85 | Steam-boiler-led | EU BREF LVIC |
| Aluminium — refinery | 0.85 | Bayer digestion/calcination steam boiler | IAI energy data |
| Aluminium — smelting | 0.65 | Anode-bake + cast-house fired furnaces | IAI energy data |
| Pulp & paper | 0.85 | Steam boiler | US DOE typical industrial steam boiler |
| Food & beverage, Textiles | 0.85 | Boiler efficiency (emissions-inversion route) | US DOE typical industrial steam boiler |

- IndustryHeat-EU subsector mapping with Climate Trace subsector

| Climate TRACE subsector | IndustryHeat-EU column | Grouped label in notebooks |
| --- | --- | --- |
| iron-and-steel | Primary Steel + Secondary Steel | Steel |
| chemicals | Chemicals | Chemicals |
| petrochemical-steam-cracking | Chemicals | Chemicals |
| cement | Cement | Cement (+lime) |
| lime | Cement (no own column) | Cement (+lime) |
| glass | Ceramics and Glass | Glass |
| pulp-and-paper | Pulp and Paper | Pulp & Paper |
| food-beverage-tobacco | Food, Beverages and Tobacco | Food, Bev & Tobacco |
| textiles-leather-apparel | Textiles and Leather | Textiles & Leather |
| aluminum | Non-ferrous Metals | Non-ferrous Metals |
| other-metals | Non-ferrous Metals | Non-ferrous Metals |

- Script pipeline description

UPSTREAM STAGES (run independently, each writes files)

├─ heat_demand/…            → facilities_2024_eu_replaceable.csv   (replaceable_heat_mwh)

├─ solar_radiation/pull_pvgis.py → hourly_profiles/*.parquet + solar summary

└─ land_availability/… (GEE + merge) → land_availability_by_facility.csv

│

▼

COMBINE  ──  flatblock_optimization/inputs/build_sites_input.py

merges the three above (by source_id) into:

→ outputs/eu_ihb_site_assessment_2024.csv   ← the single "everything" file

│

▼

OPTIMIZE ──  run_ihb_potential.py

reads  sites_csv = outputs/eu_ihb_site_assessment_2024.csv

+ Input/bnef_country_costs.csv  (country tech costs)

+ flatblock_optimization/inputs/input_heat_battery_cost.csv

+ solar_radiation/outputs/hourly_profiles/*.parquet  (read at runtime)

→ per-site LCOH / sizing results

Except for the food sector which is fitted to the JRC-IDEES/IndustryHeat-EU data.

Climate trace facilities dataset is deduplicated for sites with the same name + coordinates + identical capacity/heat and reviewed manually with sourced reasons. Overall, there were 78 duplications removed and 4 facility data updated.