Facility-level Techno-economic Potential of Electrified Industrial Process Heat in European Manufacturing

Anna Yuen1, 2, *, Amol Phadke2

1Energy and Resources Group, University of California, Berkeley
2Goldman School of Public Policy, University of California, Berkeley
*Corresponding author. Email: anna_yuen@berkeley.edu

Keywords: Industrial Decarbonization, Heat Pumps, Heat Batteries, Off-Grid Solar PV, Techno-economic analysis

Abstract

Industrial process heat has dominated industrial manufacturing emissions in the EU. Yet, there are readily available non-fossil fueled solutions that can generate process heat. Industrial heat pumps and heat batteries paired with on-site solar PV can be a potential solution. Using 2,192 facility dataset of different manufacturing sectors from Climate TRACE, each facility is modelled with co-located, off-grid solar PV with industrial heat pumps and batteries installed in 2030. Under this modelled configuration, 59% of total process heat can be decarbonized using solar PV within a 5km buffer zone around each facility site at an average levelized cost of heat at €60/MWh. After expanding the buffer zone to 15km, the share of possible decarbonized heat increased to 86% of process heat at €68/MWh. Each facility is optimized to provide the highest availability under its country’s gas prices. These results corroborate to show that a majority of the industrial process heat in the EU can be decarbonized at a rate below natural gas-powered heat costs.

1	Introduction

Industrial process heat accounts for three quarters of industrial greenhouse-gas emissions in the European Union (EU) and has been recognized as a requirement to achieve net zero emissions[1]. Different technologies are already available to electrify process heat in different temperature heat bands such as heat pumps, industrial heat batteries, electric boilers, electric arc furnaces. Electrification paired with renewable generation is widely recognized as a pathway to reduce carbon emissions and increase efficiency. Utilizing renewable generation for electrification allows for EU to use its own resources and tackle its high exposed vulnerability in fuel import dependence [2]. However, the progress to electrify process heat has been falling behind other sectors [3]. Critical barriers to electrifying industrial heat include high electricity grid prices relative to gas [3] and long lead times to connect to the electricity grid[4].

This document, saved in the “Word 97-2010” format, is a guide to using the Manuscript Template. Before submitting your final paper, check that the format conforms to this guide.  In particular, check the text to make ensure that the correct referencing style has been used and that the citations are in numerical order throughout the text. Your manuscript cannot be accepted for publication unless all formatting requirements are met.

## 1.1.3 Language, Spelling and Grammar: all papers must be written in UK English. Papers that fail to meet basic standards of literacy are likely to be rejected. Ensure that the paper is spell-checked.

## 1.1.4 Paper Length: the length of the full paper should not exceed the maximum length stipulated on the event website. 6 pages.

## 1.1.5 Page Formatting: comply with the requirements stated and demonstrated in this Manuscript Template. A PDF will be created automatically on the manuscript submission system. Authors should not copy or re-use the format of other published papers.

  - 1.1.6 Page Layout: the paper must be in double column format, 8.8cm wide with margin separation of 0.4 cm, and single spacing. This can be achieved by setting the left and right margins to 1.5 cm wide. All paragraphs must be justified, i.e. both left-justified and right-justified. This template as it stands has the correct page layout required.

2.	Methodology

We generated a facility-level data from Climate TRACE in the EU. Climate TRACE provides latitude and longitude, annual production and emissions of each facility. Combining with solar radiation and land availability in a 5km and a 15km radius around each facility, the optimization program calculates the levelized cost of heat (LCOH) at each site. The LCOH is calculated using a mixed-integer optimization framework adapted from [4], which was originally developed to size surplus interconnection capacity for renewable projects. The gas-capped, optimized LCOH at the highest availability is chosen for each site and aggregated to get the total process heat that can be decarbonized using this system and configuration.

2.1 Facility-level Process Heat Estimate

The replaceable process heat for each facility is the heat generated by fossil fuel that is used in the process of manufacturing a product (steel, food, textile, etc.). Depending on data availability, total annual production or total annual pollution is used to calculate the process heat. With total annual production, we apply a specific energy consumption (SEC) factor by sector [5] that defines the energy intensity per ton of good. With emissions total [6], we applied an emissions factor, assuming an EU averaged gas/oil share of 90%/10% [7], to get the total process heat demand. With the total process heat estimate from both SEC and emissions factor, we calculated the replaceable process heat by applying a fossils fuel share [8] to get the useful heat.

The formulas for these two processes are below:

| Calculation route | Formulas |
| --- | --- |
| SEC route | fuel_energy = production × SEC useful_heat = fuel_energy × η_process |
| Emissions route | fuel_energy = (CO₂e × combustion_share) / emissions_factor useful_heat = fuel_energy × η_boiler |
| Replaceable heat | replaceable_heat = useful_heat × fossil_share |

To validate the heat estimates, we compared against sector and country wide useful heat aggregates and determined it is within +-50% of reference data. (Supplementary graphs and plots).

2.2 Solar Radiation

Solar capacity factor, the amount of solar power output relative to nameplate, is pulled from PVGIS using ERA5 data for 2023. Hourly profiles are pulled for each facility and normalized to 1 kWp. It is then summed over a year and divided by 8760 to get the average solar capacity factor. The tilt is set to the absolute latitude and azimuth is south facing. This factor is applied to the nameplate density to get the amount of energy at each facility’s location.

2.3 Land Availability

We aggregated the square km of cropland, bare, grass and shrubland around each facility in 5 km and 15km buffer zones using google earth engine and ESA WorldCover v200 (2021) map. We applied a mask for land that is slope <= 5°C and not designated as protected areas with a buffer of 100m from Copernicus digital elevation map. While the ESA WorldCover map has 10-meter resolution, we used the zonal statistics at 30 meters for computation processing speed and to match the resolution of the slope filter.

2.4 Technology Cost Data

All costs are set in 2030-dollar values and assumes that a solar plus storage project to start constructing in 2030. Technology is assumed to have 25-year life. Capital expenditure and fixed operation and maintenance costs of solar panels and utility scale batteries are gathered from BloombergNEF (BNEF) [9] for 2030 by country. Heat pumps costs by temperature band [10] and industrial heat batteries [11] are in 2021 and 2023 dollars respectively. Both are CPI- (Bureau of Labor Statistics, 2025) to 2030 which applies a factor of 1.31 and 1.17 respectively. Country specific costs from BNEF and cost of capital rates (IRENA) are used when direct country costs are available and proxied by peer mean income country groups (HIC, LMIC, etc.) when not available.  To annualize costs across a project’s lifetime, we used this LCOH formula:

CRFc=rc1+rcN1+rcN-1, N=25

Cann=Csolarann+CLi-ionann+CIHBann+CHPann

Csolarann=CAPEXcPV⋅1000⋅CRFc+FOMcPVS*

CLi-ionann=CAPEXcbatt4⋅1000⋅CRFcELi*+FOMcbatt Pdis,Li*

CIHBann=b∈IHBCAPEXin Pch,b*+CAPEXout Pdis,b*+CAPEXE Eb*⋅1000⋅CRFc

CHPann=b∈HPCAPEXbHP⋅1000⋅CRFc+FOMbHP⋅1000Qb*

LCOHserved=CannQserved,

LCOHtotal=CannQdemand

Where rc is country WACC, S*, E*, P* are optimized capacities, Qb* is HP thermal capacity sized to served heat in band b, and Qserved=Qdemand-Qunserved. Unserved energy is not costed (VoLL = 0).

2.5 Gas Heat Benchmark

Each LCOH is compared against a gas heat benchmark to get the highest availability of electrified heat generated below the gas heat price. Gas heat prices are gathered from Eurostat’s non-household gas for 2030 (nrg_pc_203)[12]. A general gas boiler efficiency of 0.85[13] is applied to the gas price.

The EUA carbon tax of €79/tCO₂ is assumed and added to the gas heat cost.

cheat,cgas=pcgasη+pEUA⋅EFNGη

η&=0.85&&(boiler efficiency)pEUA&=79 EUR/tCO2&&(EUA carbon price assumption)EFNG&=0.202 tCO2/MWhfuel&&(gas CO2 emission factor)

2.6 Optimization – System Sizing

To get the optimized system size to minimize cost, we sized the co-located solar panels with heat pump or heat battery solution using a linear program (HiGHS). Replaceable heat below 200°C is allocated to heat pump with li-ion battery and above 200°C is allocated to industrial heat battery. Both of those solutions will pipe heat into the industrial facility. The linear program is defined below:

min 1QdemcSS+cELiELi+cPLiPdisLi+b∈BIHBcinPbch+coutPbdis+cEEb

c&∈1,…,K&&representative days (K=16 default)t&∈1,…,H&&hours (H=24)b&∈BHP&&COP>1: <100 °C, 100--200 °Cb&∈BIHB&&COP≤1: ≥200 °CηLi&=RTELi, ηIHB=RTEIHBRTELi&=0.90, RTEIHB=0.95,\quadhmax=16 h

Constraints

0≤S≤Smax=Aland×50 MWdc/km2

ELi≤hmax PdisLi

b∈BHPdb,c,t+pc,tch,Li+b∈BIHBpb,c,tch≤S gc,t

with t-1≡H when t=1.

0&≤sc,tLi≤ELi0&≤pc,tch,Li≤PchLi0&≤pc,tdis,Li≤PdisLipc,tdis,Li&=b∈BHPfb,c,t

COPbdb,c,t+fb,c,t+ub,c,t=Lb ϕt

pb,c,tdis+ub,c,t=Lb ϕt

sb,c,t-sb,c,t-1-COPb ηIHB pb,c,tch+1ηIHB pb,c,tdis=0

0&≤sb,c,t≤Eb0&≤pb,c,tch≤Pbch0&≤pb,c,tdis≤Pbdis

c=1Kwct=1Hb∈Bub,c,t≤1-α Qdem

S, ELi, P∙Li, Eb, Pbch, Pbdis, db,c,t, fb,c,t, ub,c,t, pc,t∙, sc,t∙≥0

\captionFacility-levelHiGHSLPrepresentativePVGISdays.Sharedsolar$S$feedsHPbands$<$200 °C;electricLi-ionbufferandIHBbands$≥$200 °C;resistivecharge$→$thermalstore.$gc,t$isnormalizedsolaryieldkW/kWp;$Lb$isflatMW$th$bandload;$ϕt=1$bydefaultflatdiurnalshape.$wc$aredayclusterweights$cwc≈365$.Round-tripefficienciesenteras$η=RTE$.Land-limitedsitesthatcannotmeet$α=90%$firstminimizeunservedenergy,thenminimizeannualizedcostatthatservedlevel.UnservedenergyisnotpricedinLCOH$VoLL=0$

Utilizing the hourly solar profiles and the flat-load heat demand, we optimized for availability levels up to 10 to 100% in 10% increments. Then we select the availability level below the gas heat cost benchmark. Below is an example of a steel plant in Germany where the decarbonized solution could provide 80% availability before LCOH exponentially increases and reach above the gas benchmark.

After sizing the solar panels and batteries for both the 5km and 15km radius, the total availability for each plant is averaged and aggregated to get the total amount of heat decarbonized and the heat weighted LCOH at the two buffer zones.

3	Results

## 3.1 Figures

Graphics may be full colour but make sure that they are appropriate for print (black and white) and online (colour) publication. Coloured line graphs should use dotted or dashed lines, or shapes to distinguish them apart in print. Examples of graphs in acceptable formats are given in Fig 1. Each figure should be explicitly referred to in numerical order and should be embedded within the text at the appropriate point. A maximum of four subfigures are allowed per figure.

a

b

3.1.1 Figure Captions: two examples of figure captions in an acceptable format for Fig 1.are given below:

Fig. 1 (a) Sample graph with pink (dotted), blue (solid) and black (dashed) lines, (b) Sample graph with black (diamond), black (triangle) and black (square) markers

or

Figure 1 Sample graph

(a) Subfigure 1

(b) Subfigure 2

Figure captions must be below the figure, in 10pt font and should ideally consist of one sentence.  If a figure has subfigures, all subfigures should also have a caption and should be identified by letters, e.g. (a), (b), (c), (d) as shown above.

## 3.2 Tables

Tables should be formatted as shown in the example below with no column lines, unless these are essential for clarifying the content of the table. Row lines can be used to distinguish the column headings from the content of the table.

Table 1 Example table

| Column heading | Column heading two | Column heading three |
| --- | --- | --- |
| Row 1 | Row 1 | Row 1 |
| Row 2 | Row 2 | Row 2 |

  - 3.2.1 Table Captions: tables must be numbered and cited within the text in strict numerical order.  Table captions must be above the table and in 10 pt font.

## 3.3 Mathematical equations

- Equations should fit into a two-column print format and be single spaced.

- When writing mathematics, avoid confusion between characters that could be mistaken for one another, e.g. the letter 'l' and the number one.

- Vectors and matrices should be in bold italic and variables in italic.

- If your paper contains superscripts or subscripts, take special care to ensure that the positioning of the characters is unambiguous.

- Exponential expressions should be written using superscript notation, i.e. 5x103 not 5E03.

- A multiplication symbol should be used, not a dot.

- Refer to equations using round brackets e.g. (1)

## 3.4 Units

- Use SI (MKS) units only and avoid spelling the unit in full instead of using the shortened notation e.g. use kJ not kilo Joules.

- If for any reason you must use mixed units, the units used for each quantity in an equation must be stated.

- Place a zero before decimal points: “0.10” do not put “.10.”

4	Conclusion

Submissions should always include the following sections: an abstract; an introduction; a conclusion and a references section. If any of the above sections are not included the paper may be asked to add the relevant section or be rejected.

5	Acknowledgements

Acknowledgements should be placed after the conclusion and before the references section. Details of grants, financial aid and other special assistance should be noted.

6	References

6.1 Referencing Style

You should number your references sequentially throughout the text, and each reference should be individually numbered and enclosed in square brackets (e.g. [1]).

If the number of authors on a reference is greater than three list the first three authors followed by et al.

Ensure that all references in the Reference list are cited in the text and vice versa. Failure to do so may cause delays in the publishing of your paper, should you be accepted.

Ensure that you provide as much information as possible to allow the reader to locate the article concerned. This is particularly important for articles appearing in conferences, workshops and books that may not appear in journal databases.

Do not include references for papers that have been submitted and not accepted for publication. Papers that have been accepted for publication are allowed as long as all information is provided.

Provide all author name(s) and initials, title of the paper, date published, title of the journal or book, volume number, editors (if any), and finally the page range. For books and conferences, the town of publication and publisher (in parentheses) should also be given.

Example References

6.2 Referencing Examples

Journal articles

[5] Smith, T., Jones, M.: 'The title of the paper', IET Syst. Biol., 2007, 1, (2), pp. 1–7

[6] Borwn, L., Thomas, H., James, C., et al.:'The title of the paper, IET Communications, 2012, 6, (5), pp 125-138

Conference Paper

[7] Jones, L., Brown, D.: 'The title of the conference paper'. Proc. Int. Conf. Systems Biology, Stockholm, Sweden, May 2006, pp. 1–7

Book, book chapter and manual

[8] Hodges, A., Smith, N.: 'The title of the book chapter', in Brown, S. (Ed.): 'Handbook of Systems Biology' (IEE Press, 2004, 1st edn.), pp. 1–7

[9] Harrison, E.A., and Abbott, C.: 'The title of the book' (XYZ Press, 2005, 2nd edn. 2006)

Patent

[11] Brown, F.: 'The title of the patent (if available)'. British Patent 123456, July 2004

Report

[10] IET., 'Report Title' (Publisher, 2013), pp. 1-5

[12] Smith, D., Hodges, J.: British Patent Application 98765, 1925

Standard

[14] BS1234: 'The title of the standard', 2006

Thesis

[13] Abbott, N.L.: 'The title of the thesis'. PhD thesis, XYZ University, 2005

Websites

[1] ‘Author Guide - IET Research Journals’, http://digital-library.theiet.org/journals/author-guide, accessed 27 November 2014

[2] ‘Research journal length policy’, http://digital-library.theiet.org/files/research_journals_length_policy.pdf, accessed 27 November 2014

[3] ‘ORCID: Connecting research and researchers’, http://orcid.org/, accessed 3 December 2014

[4] ‘Fundref’, http://www.crossref.org/fundref/, accessed 4 December 2014

Additional Information

I)	Submission Specification for Conference Papers

This ‘Submission Specification for Conference Papers’ document provides instructions for the submission of Conference Papers for publication on the IET Digital Platform.

This document is to be used in conjunction with the ‘Manuscript-template.docx’.

The Conference Paper is to be prepared in MS Word format and then printed to PDF.

# 1.	Submission requirements

Conference papers must satisfy the following requirements:

### 1.1	Manuscript Submission

1.1.1	Originality: Is the work scientifically rigorous, accurate and novel? Does the work contain significant additional material to that already published? Has its value been demonstrated?

1.1.2	Plagiarism: All conference papers submitted to the IET for publication must record original work which has not been published previously. Any alleged cases of plagiarism will be dealt with according to the ‘IET Policy in Relation to Plagiarism, Infringement of Copyright and Infringement of Moral Rights and Submission to Multiple Publications’.

1.1.3	Multiple submissions: The IET does not permit manuscripts included in its conference proceedings to be simultaneously under review for another conference or publication.

Once a case of multiple submissions has been established, the paper/s in question will be immediately declined for publication by the IET.

1.1.4	Relevance: Is the material appropriate to the scope of the conference to which it is submitted?

1.1.5	Clarity: Is the English clear and well written? Poorly written English may obscure the scientific merit of your paper. Are the ideas expressed clearly and concisely? Are the concepts understandable?

### 2.	Post-print Policy

2.1.1	The author may post post-prints of their conference papers published by the IET on repositories, servers and websites of any sort, provided that these servers are operated by the author's institution or the funding body contributing to the research. To comply with funding requirements, authors may also deposit their conference papers in repositories (or mirror sites) designated by the funding body.

The conditions attached to this are as follows:

2.1.2	Access to such servers is not for commercial use and does not depend on payment of access, subscription, or membership fees.

2.1.3	The following wording clearly appears on the front page of the post-print:

"This paper is a post-print of a paper submitted to and accepted for publication in Conference Proceedings of conference title and is subject to Institution of Engineering and Technology Copyright. The copy of record is available at IET Digital Library"

2.1.4	The post-print must be the author's version and not the IET version/PDF.

2.1.5	The post-print must not be posted prior to publication of the paper by the IET and when posted any preprint version should be removed.

### 3.1	Copyright

3.1.1	An author submitting a paper should ensure that he or she has the right to publish the paper and that it contains nothing defamatory. The IET will assume that all co-authors have agreed to the submission of any paper received.

3.1.2 	Your conference paper must be accompanied by a signed licence to publish.

3.1.3	Copyright: Unless the paper is either "United Kingdom Crown copyright" or "a work of the US Government and in the Public Domain", the copyright remain with the author(s).

3.1.4	Right to publish: An author submitting a paper should ensure that he or she has the right to publish the paper and that it contains nothing defamatory. The IET will assume that all co-authors have agreed to the submission of any paper received.

### 4.1	Permissions to Reproduce

4.1.1	All authors must secure permission from the copyright holder for reproducing previously published text, figures and tables. The source of the reproduced material must be given in full and the words 'Reproduced by permission of .....' included with the illustration.

# II)	PDF format requirement

All PDFs shall be set as follows:

Version: Adobe PDF Version 1.5-1.8 (Acrobat 5-8)

Fonts: All fonts must be 'embedded subset'.

Document Properties:

Title: Full title of the paper.

Authors: All contributing author names to be included.

Subject: Full conference name and abbreviation.

Keywords: Keywords as set in the paper at Item 2, Keywords.

# III)	IET Digital Library and Inspec Indexing requirements

Papers that will be submitted for publication on the IET Digital Library, and subsequent indexing in Inspec, IEEE Xplore and EI Compendex, must be submitted in the following format:

All PDFs must be in the format as specified as stated in Paragraph 2 above (PDF format requirement).